Title: Introduction to Microprocessor Kit 8086.

# Abstract:

This lab focuses on the introduction to the 8086-microprocessor kit, its architecture, and the process of writing and executing instructions in memory. The 8086 microprocessor is a 16-bit processor designed by Intel, which operates on a segmented memory model and supports a variety of addressing modes. The primary objective of this experiment is to familiarize students with the 8086-microprocessor kit, understand its hardware and software components, and learn how to input assembly instructions into memory for execution. By performing basic arithmetic, logical, and data transfer operations, the lab demonstrates how instructions are stored, accessed, and executed in the system. This hands-on experience is crucial for understanding low-level programming and the fundamentals of microprocessor-based system design.

# Introduction (Theory):

The 8086 Microprocessor is a 16-bit processor developed by Intel, which serves as the foundation for modern computing architectures. It operates in minimum mode (single processor system) or maximum mode (multiprocessor system) and is capable of addressing up to 1 MB of memory. The microprocessor consists of various functional units, including the Bus Interface Unit (BIU) and the Execution Unit (EU), which work together to fetch, decode, and execute instructions.

The 8086 microprocessor follows the Von Neumann architecture, where both data and instructions share the same memory. It supports segmented memory architecture, which divides memory into Code, Data, Stack, and Extra Segments, allowing efficient memory management. The processor uses 20-bit address lines and 16-bit data lines, enabling it to access up to 1 MB of memory (2^20 bytes).

Key components of the 8086-microprocessor:

General-purpose registers: AX, BX, CX, DX

Segment registers: CS, DS, SS, ES

Pointer and index registers: SP, BP, SI, DI

Instruction Pointer (IP): Holds the address of the next instruction.

Flags Register: indicates the status of the processor after Execution.

# Material:

| Hardware | Software |
| --- | --- |
| - 8086 Microprocessor Trainer Kit - Computer (32-bit Windows PC) | - MASM (Microsoft Macro Assembler) - Text Editor (Notepad) - Command Prompt (CMD) |

Method:

## Code:

## Execution

# Result:

Theoretical:

| Instruction | AX |  | BX |  | CX |  | DX |  | IP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AH | AL | BH | BL | CH | CL | DH | DL |  |
| Initial | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 1000 |
| MOV AX, 1234H | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 1003 |
| MOV BH, 80H | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 1005 |
| MOV BL, AH | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 1007 |
| MOV CX, 1144H | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 100A |
| MOV DX, 0FFFH | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 100D |
| ADD DX, 1 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 1010 |
| SUB AX, CX | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 1012 |
| DEC BH | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 1014 |

Practical:

Discussion: