Question 1

A warehouse uses a 48 V DC power supply to operate its emergency lighting system. Each lighting unit contains three resistive lamps connected in series, each lamp having a resistance of 8 Ω. There are five identical lighting units connected in parallel across the supply. The main supply cables have a total resistance of 1 Ω.

Determine:

- The resistance of one lighting unit

- The equivalent resistance of all five units in parallel

- The total circuit resistance including the cable resistance

- The total current drawn from the 48 V supply

- The current flowing through each lamp

Question 2

A robotic vehicle is powered by a 36 V DC battery. The control circuit consists of: Two resistors (6 Ω and 12 Ω) connected in series. This series combination is connected in parallel with a 9 Ω resistor. The internal resistance of the battery is 0.6 Ω

Determine:

- The equivalent resistance of the external circuit

- The total circuit resistance

- The current supplied by the battery

- The current through each branch of the parallel network

- The power dissipated in the 9 Ω resistor

Question 3

A telecommunication equipment room uses a battery-backed supply to power multiple monitoring devices. At a distribution panel, the supply feeds a junction point (Node A) where current splits into three branches:

- Branch 1: A cooling fan connected from Node A to ground through a 4 Ω resistor (R₁)

- Branch 2: A status indicator panel connected from Node A to ground through a 6 Ω resistor (R₂)

- Branch 3: A backup control module connected to a 12 V DC reference battery through an 8 Ω resistor (R₃)

The system engineer needs to analyze the circuit to ensure proper current sharing and to verify whether the 12 V battery is being charged or discharged.

Question 4

An pressure sensor module uses a DC amplifier to condition low-level sensor signals before sending them to a PLC (Programmable Logic Controller). To ensure stable operation across temperature variations, the amplifier’s transistor biasing network is powered by a 30 V regulated DC supply and stabilized using a 10 V precision reference source.
Additionally, the sensor element injects a small bias current into the amplifier input node due to its internal electronics.

At Node , the following connections exist:

- R₁ = 10 kΩ from the 30 V supply to Node B

- R₂ = 5 kΩ from Node B to ground

- R₃ = 15 kΩ from Node B to a 10 V reference source

- A current source of 1 mA injects current into Node B (representing sensor output bias current)

- Find The voltage at Node B and current through each path.

Question 5

An conveyor control panel uses two DC control loops to operate relays and actuators.
To reduce wiring complexity, the two control loops share a current-limiting resistor.

- Loop 1 controls a motor relay

- Loop 2 controls a safety interlock relay

- The shared resistor represents a common sensing resistor used to monitor current

### In Motor Relay, there is a Voltage source: 20 V and Resistors: 4 Ω and 6 Ω (shared) and in Safety Interlock Loop there is a Voltage source: 10 V, Resistors: 5 Ω and 6 Ω (shared) Find the current flowing through each loop.

Question 6

In an electronics laboratory, a DC power supply feeds a test circuit used to evaluate resistor networks and power sharing between multiple sources.

- Loop 1 simulates a primary power rail

- Loop 2 represents a load balancing network

- Loop 3 simulates a secondary backup supply

Shared resistors model common wiring paths.

- Voltage source: 18 V

- Resistors: 3 Ω, 7 Ω (shared with Loop 2)

- Resistors: 7 Ω (shared with Loop 1), 5 Ω (shared with Loop 3)

- Voltage source: 12 V

- Resistors: 5 Ω (shared with Loop 2), 4 Ω. Find currents through each path.

Exercise

An underground parking lot uses LED floodlights powered by a 24 V DC supply from a battery bank. Each floodlight module has two identical LEDs in series inside its housing, each with a resistance of 5 Ω There are four floodlight modules connected in parallel to the supply. The cables from the battery bank to the lights have a total resistance of 0.5 Ω The maintenance engineer needs to determine: The total current drawn from the battery. A sine wave passes through a diode /half-wave rectifier, then through an RC circui