### Definition

A carbon microphone is a transducer that converts sound energy into electrical signals using carbon granules.

### Working Principle

- Sound waves hit the diaphragm.

- The diaphragm vibrates.

- The movable electrode compresses and releases the carbon granules.

- The density of the carbon granules changes.

- As density changes, electrical resistance changes.

- This produces a changing electrical current that carries the voice signal.

### Chain Reaction (Slide Based)

Sound Waves → Diaphragm Vibrates → Carbon Granules Compress/Expand → Resistance Changes → Electrical Signal Produced

# 3. Why is a Small Amount of Sidetone Necessary? (5 Marks)

### Definition

Sidetone is hearing a small part of your own voice in the receiver while talking.

### Why is it Necessary?

- It lets the caller know the phone is working.

- It helps maintain a natural speaking level.

- Without sidetone, people tend to shout.

- Too much sidetone makes people speak too softly.

- Therefore, a balanced (small) amount of sidetone gives the best communication.

# 4. Low & High Sidetone (Anti-Sidetone Circuit) (5 Marks)

### Low Sidetone (Transmitting)

When we speak,

- Current in L1 and L2 flows in opposite directions.

- Magnetic fields cancel each other.

- Very little current is induced in L3 (Receiver).

- We hear only a small amount of our own voice.

### High Sidetone (Receiving)

When another person speaks,

- Current in L1 and L2 flows in the same direction.

- Magnetic fields add together.

- Strong current is induced in L3.

- Incoming voice is heard clearly.

### Sidetone

Definition:
Sidetone is the speaker's own voice heard in the receiver (earpiece) while talking.

### Echo

Definition:
Echo is the reflected voice that returns to the speaker due to impedance mismatch in the transmission path

## Why is a Switching System Important? (5 Marks)

- Establishes Connection – Connects the caller with the receiver.

- Routes Calls – Selects the correct path for the call.

- Efficient Resource Use – Shares network resources efficiently.

- Supports Many Calls – Allows many users to communicate at the same time.

- Disconnects Calls – Releases the connection after the call ends so it can be used again.

## Caller ID (Easy 2–3 Marks)

Definition:

Caller ID (CID) is a telephone service that shows the caller's information (name and/or phone number) on the receiver's phone before it rings.

Easy Definition:

Caller ID is a service that displays the caller's phone number or name before the phone rings.

## How Caller ID Works (Easy 5 Marks)

- The exchange detects an incoming call.

- It collects the caller's information (name/number).

- The information is sent using FSK (Frequency Shift Keying).

- The receiver's phone decodes the data.

- The phone displays the caller's details before ringing.